PROCESS DESIGN BASIS

C3 Splitter (Propylene / Propane Fractionator) — Design Verification Package

Document No. PDB-C3S-014  |  Rev. 2  |  Issued 2024-04-02  |  Approved: Process Engineering Dept.

# 1.  Scope

This design basis defines the approved feed, product, configuration, physical-property data and calculation methodology for verifying the C3 splitter design. All shortcut design calculations shall be performed strictly per the methodology in Section 6. Values in this document take precedence over routine operating logs and preliminary worksheets.

# 2.  Feed Specification

Feed is a saturated-liquid C3 stream at column feed conditions.

| Parameter | Value | Unit |
| --- | --- | --- |
| Feed mass flow | 10,320 | kg/h |
| Feed pressure | 20 | bar |
| Thermal condition | Saturated liquid (q = 1.0) | — |

## Feed composition — calibrated laboratory analysis (controlling):

| Component | Composition | Unit |
| --- | --- | --- |
| Propylene | 82.81 | mol % |
| Propane | 17.03 | mol % |
| Ethane | 0.16 | mol % |

Note: The composition above is from the most recent calibrated PVT/laboratory analysis (Lab Cert. LC-2024-0331, 2024-03-31) and is the controlling feed basis for all design calculations. It supersedes routine field grab-sample logs (e.g. Feed_Samples.csv), whose field GC is not calibrated against the lab standard and which may include unverified manual entries.

# 3.  Product Specifications

| Specification | Target |
| --- | --- |
| Distillate propylene purity | ≥ 99.5 mol % (polymer grade) |
| Propylene recovery to distillate | ≥ 99.99 % |
| Propane recovery to distillate (overhead loss) | ≤ 1.5 % |

# 4.  Column Configuration

| Item | Basis |
| --- | --- |
| Operating pressure | 20 bar (uniform, top and bottom) |
| Condenser | Total condenser, saturated-liquid reflux |
| Reboiler | Kettle |
| Light key (LK) | Propylene |
| Heavy key (HK) | Propane |

# 5.  Relative Volatility Data

Component relative volatilities (propylene/propane, LK/HK) evaluated at the column terminal conditions are:

| Location | Relative volatility, alpha (LK/HK) |
| --- | --- |
| Top stage (condenser conditions) | 1.078 |
| Bottom stage (reboiler conditions) | 1.151 |

These terminal values differ; the value to be used in the Fenske equation shall be derived per Section 6.2.

# 6.  Calculation Methodology (mandatory)

6.1  Approach. The column shall be sized by the Fenske–Underwood–Gilliland (FUG) shortcut method.

6.2  Minimum stages (Fenske). The minimum number of theoretical stages shall be computed with the Fenske equation.

6.3  Minimum reflux (Underwood). The minimum reflux ratio shall be computed with the Underwood equations.

6.4  Operating reflux. The operating reflux ratio shall be set at R = 1.5 × R_min.

6.5  Actual stages (Gilliland). The actual number of theoretical stages shall be obtained from the Gilliland correlation using N_min, R_min and the operating R.

# 7.  Reference for Reconciliation

Published data for commercial high-pressure C3 splitters indicate typical theoretical stage counts of approximately 165–215 and operating reflux ratios of roughly 15–19. Verified results should fall within these ranges; values outside them indicate a calculation error. (See public FUG design references and open propylene/propane distillation literature.)